INTRODUCTION

The world is embracing Internet of Things (IoT) applications. Billions of internet-connected devices are capable of sensing, communicating, interacting, computing and actuating, and are set to become even more integrated into our daily lives. By 2022, around 18 billion out of 29 billion connected devices will be related to IoT. While short-range IoT devices are connected via technologies such as Wi-Fi and Bluetooth, wide-area IoT devices are connected to the internet using cellular connections or Low-Power Wide-Area Networks (LPWAN), such as Sigfox, LoRa, NB-IoT or LTE-M.

In this relatively new but growing market of wide-areaIoT, positioning is key to provide non-stationary devices knowledge of their spatial location. Terrestrial connectivity solutions often offer energy-efficient localisation based on their ground infrastructure. However, they lack the accuracy required to fully support the needs of all IoT applications. In contrast, for outdoor applications, Global Navigation Satellites Systems (GNSS) provide the extremely precise, robust and ubiquitous positioning and timing information that the connectivity-based technologies lack. Despite these evident advantages, some IoT devices still use inaccurate infrastructure-based methods, due to stringent energy consumption requirements, which remain a challenge for standard GNSS chipsets.

However, there has been a successful push to significantly reduce GNSS energy consumption over the last few years, thanks to rapid advancements in receiver technology and the arrival of several innovative techniques. Consequently, GNSS is increasingly attractive for low-power IoT applications and is paving the way for new applications and markets. Widely available solutions such as Assisted GNSS (A-GNSS) or long-term ephemeris predictions, and novel cloud-based approaches will lead to an increase in the uptake of GNSS for low-power IoT devices.

Depending on the chosen solution, either no connectivity to a network at all, or merely a downlink connection is required to determine position. Novel solutions often require up- and downlink connectivity to determine the position in the cloud. Along with so-called “snapshot” techniques, the transmission of pseudoranges to the cloud for subsequent outsourced position calculation is an example of these innovative approaches

This white paper provides an overview of relevant GNSS technologies for low-power IoT, including those that require hybridisation with different connectivity solutions.

GNSS AND ITS ROLE FOR LOW-POWER IOT

With millions of moving interconnected devices in the IoT environment, many applications require or benefit from knowing the location of an individual device. Low-power geolocated IoT devices can already be found in various industry verticals today and will enter multiple new fields in the future. In manufacturing and the supply chain, tracking devices make it possible to locate containers, pallets, and various other objects. They also allow autonomous logistic trains to be guided on the most efficient routes through factories and will enable better planning of arrival windows for shipments. Furthermore, geolocated devices will enable more efficient use of resources and goods in smart city applications. Besides enabling green mobility solutions, like rental bikes, positioning solutions can optimise waste management and help to tackle problems like theft and vandalism more efficiently.

Positioning also plays an important role in healthcare and the leisure industry. For example, wearables can be used by sports enthusiasts to track their activities, but can also enable the elderly to send a geolocated signal in case of an emergency. In addition, ubiquitous positioning is already revolutionising traditional industries like agriculture, as small beacons support livestock and machinery tracking. For many of the IoT applications mentioned, precise, ubiquitous and energy-efficient positioning of the mobile objects is a key enabling element.

What positioning solution best suits your needs?

LPWAN-BASED SOLUTIONS

IoT devices can be connected to the internet through various wireless telecommunication networks. These may already offer positioning services based on Received Signal Strength Indication (RSSI), Time Difference of Arrival (TDOA), or Observed Time Difference of Arrival (OTDOA) approaches. Although such geolocation technologies might be sufficient for some applications, they all share the common disadvantage that they can only be used in close proximity to network base stations and are usually only able to offer low positioning accuracy in the order of hundreds or even thousands of meters.

GNSS-BASED SOLUTIONS

In contrast, GNSS enables precise and reliable ubiquitous positioning all over the globe – independent of telecommunication network infrastructure. GNSS positioning depends on radio signals emitted by satellites in medium Earth orbit. Each satellite broadcasts signals that are picked up by receivers on the ground with a delay corresponding to the time it took the individual signals to travel from the satellite to the receiver. These signals also include information on the satellite’s position. The receiver’s position is then determined by trilateration across at least 4 satellites (for x, y, z, and time dimensions).

Multiple GNSS constellations The accuracy performance of GNSS is a function of the satellites-to-receiver geometry quantified by the Geometric Dilution of Precision (GDOP) factor. A larger number of satellites in view results in a better GDOP (improved position accuracy) and higher signal availability, particularly in urban environments where the line of sight to the satellites might be partially obscured by buildings. Thus, multi-constellation receivers are always advisable so that satellites from all the available systems – Galileo, GPS, GLONASS and BeiDou- are leveraged

Multiple frequencies While standard GNSS receivers utilise signals transmitted in one frequency band (L1/E1 band) and allow for position accuracy of a few meters, processing GNSS signals in multiple frequencies can provide significant benefits. Not only does this option allow for better positioning accuracy by cancelling out some propagation errors, it also provides a better protection against local disturbances such as interferences or multipath. Multi-constellation multi-frequency receivers are therefore advisable for high end IoT applications. However, their higher performance currently comes at the cost of an increase in overall energy consumption and a higher price compared to a multi-constellation single-frequency receiver

Galileo differentiators for IoT

As the European GNSS, Galileo offers users of its Open Service (OS) a reliable and free of charge service under civil control. Interoperability with other satellite navigation systems enhances availability, robustness and accuracy for multi-constellation users around the world. These benefits are acknowledged by both industry and users. Consequently, virtually all new receivers are capable of receiving Galileo signals

A UNIQUE AUTHENTICATION FUNCTION

For commercially sensitive IoT applications, the soon-tobe-launched unique Open Service Navigation Message Authentication (OS-NMA) will improve resilience against spoofing and add security by allowing users to verify that a navigation message comes from a Galileo satellite and not a potentially malicious source.

THE HIGH ACCURACY SERVICE

While the accuracy of multi-constellation single-frequency GNSS is sufficient for many IoT applications, those that prioritise higher accuracy over energy consumption and cost can benefit from multi-frequency GNSS. Applications requiring even higher accuracy will benefit from Galileo’s forthcoming High Accuracy Service (HAS) in the near future, delivering centimetre level accuracy

Optimising GNSS position determination for IoT devices

Although GNSS provides highly accurate and ubiquitous position and time information, the relatively high energy consumption of the technology in its typical use case1 does not align well with the stringent constraints of battery-powered IoT devices which are often expected to function intermittently for multiple years without charging. Fortunately, multiple techniques exist to overcome this inconsistency.

Receiver duty cycling

Recognising that positions are in many cases required on demand rather than continuously, duty cycling consists in powering off all the components of a GNSS receiver except those required to react to a wake-up call, thus drastically reducing its power consumption. This technique is currently implemented in virtually all mass market receivers.

Extended and autonomous ephemeris prediction

The high energy consumption of GNSS receivers mainly comes from the so-called acquisition phase, covering both the signal acquisition and the navigation message retrieval. Broadcast by the GNSS satellites, the navigation messages include parameters needed to compute the satellites’ positions and clock corrections. Even though such messages are small in size, their low transmission rate results in long download times during which the receiver must remain fully powered: for each so-called cold start, about 30 seconds elapse until the position is determined. To solve this problem, the navigation message can be obtained from an alternative source:

• Either computed by the receiver autonomously, based upon past data, or

• Received via the telecommunication network2 , possibly with an extended validity to reduce the frequency of the downloads.

Assisted GNSS

GNSS assistance consists in supplying the GNSS receiver through a communication network with data that will help during the acquisition phase:

• Acquisition assistance data includes coarse timing and Doppler information, used to shorten the power intensive signal acquisition;

• Clock and ephemeris assistance data replace the broadcast navigation message, as seen above.

Thus A-GNSS minimises the overall GNSS energy consumption by tackling its two main sources, albeit at the cost of more demands on the communications link needed to receive the assistance data.

Snapshot processing

Going forward, snapshot techniques can reduce energy consumption even further as they make it possible to determine the position by using only a minuscule interval of a GNSS signal that is subsequently processed with the help of assistance data to retrieve pseudorange information and compute the receiver position. These techniques however come at the cost of a reduced sensitivity and accuracy, and a proper balance must be found.

Cloud processing

Although the various implementations of assisted GNSS already reduce energy consumption, downloading assisted data from an external network is not always possible, or some applications have even stricter energy requirements that cannot be met by such techniques alone. To further reduce energy consumption in a significant way, a change of paradigm in the way the position is calculated is required. Instead of performing all GNSS tasks in a single receiver, energy-hungry functions, for example the position determination based on retrieved pseudoranges can be “outsourced” to the cloud, where sufficient energy, processing power and clock and ephemeris data are available in virtually unlimited quantity.

LPWAN as an enabling connectivity technology for GNSS solutions in IoT

Most energy-efficient GNSS techniques require the exchange of data with a network to determine the device position. For low-power IoT applications, different Low-Power Wide-Area Networks (LPWAN) are the connectivity solution of choice, as they combine long-range, low-power with cheap connectivity and are therefore ideally suited to affordably connect battery-powered geolocated IoT devices on a large scale. The low data rates offered are often sufficient for many applications, but can result in difficulties when combined with novel GNSS positioning techniques, particularly when considering the severely limited downlink capability of proprietary solutions. LPWAN vary significantly in their maximum up- and downlink capacity as well as energy efficiency and can be separated into proprietary and cellular-based networks. The two highly energy-efficient proprietary LPWAN Sigfox and LoRa both operate in the unlicensed ISM (Industrial, Scientific and Medical) radio bands. Radio regulations in these bands limit the amount of daily uplink, and especially downlink data, allowing only for slow and infrequent data transmissions. The Sigfox network already covers large parts of Europe, and users can purchase subscriptions to connect IoT devices to the network. LoRa additionally offers users the possibility to deploy their own private network gateways. Both LoRa and Sigfox are very slow compared to cellular LPWAN. The cellular LPWAN NB-IoT and LTE-M are standardised by 3GPP and are based on widely available LTE technology and infrastructure. NB-IoT allows energy-efficient communication at medium data rates, while LTE-M enables fast communication at the cost of shorter battery life and more complex and expensive hardware. As both technologies operate in the licensed LTE bands, data throughput is not restricted by local radio regulations and a high quality of service and more reliable radio communication can be ensured

Assisted data from external network

Instead of obtaining the navigation message directly from the satellite during a cold start, A-GNSS makes it possible to acquire the required information, including ephemerides data and clock corrections, from an external source such as an LPWAN or cellular network. Besides clock and ephemerides data, assistance messages also include support data (coarse time and position, Doppler) to speed-up the signal acquisition sequence of the GNSS receiver. Assisted GNSS (A-GNSS) is already widely used for smartphones and has found its way into many cellular-based LPWAN modules already. When utilising this technique, the user receives two major benefits: • Faster position fix and therefore reduced energy consumption; • Higher receiver sensitivity improves performance in difficult environments, such as indoor and urban. These benefits make usage of the technique highly advisable for any IoT device that has access to a fast enough network. Unfortunately, with assisted data packages of several kilobytes in size, A-GNSS cannot be used with every LPWAN, as the downlink capacity is often highly limited. This is especially true for proprietary LPWAN operating in the unlicensed ISM bands, as they must comply with radio regulations. On the other hand, these regulations are not applicable to the downlink of cellular LPWAN, such as NB-IoT and LTE-M, making the transmission of assisted data possible. The technique is already implemented in several cellular-IoT modules and can be used today. The validity of the transmitted data is in the range of multiple hours, after which the download of the new assistance data is required.

To further decrease energy consumption and increase autonomy, several companies offer the provision of assistance data with a validity of up to multiple weeks. While minimising the download frequency of data, resulting in lower energy consumption, this solution comes at the cost of decreased position fix accuracy with time. As the future orbital and clock parameters of the GNSS satellites are unknown and prone to perturbations, the extended ephemeris data provided can only be estimated based on models. With the size of the extended data often being proportional to the validity, the validity duration of the extended ephemeris package should be chosen according to the individual accuracy requirements, the network downlink capacity, and the use case